King Fahd University of Petroleum and Minerals

College of Computer Science and Engineering

COE306: Introduction to Embedded Systems

COE306 Project

Automatic A/C Controller

By:

Osama Bujwaied

201661700

For:

Dr. Hazim Selmi

Dr. Abdulaziz Tabakh

2 May 2020

Table of Contents

1. OBJECTIVE 	3

2. PART LIST 	3

3. BACKGROUND	3

3.1 Arduino Uno	3

3.2 DS18B20 Temperature Sensor	3

3.3 Potentiometer	3

3.4 I2C Lcd Display	3

4. STEPS OF PROJECT	3

4.1  Connection And Programing Temperature Sensor	3

4.2  Reading User Temperature Request	3

4.3  Assigning Light To Show A/C Condition 	3

4.4  Configure I2C LCD Display	3

4.5  Overall Design	3

5. LINK TO VIDEO	3

- OBJECTIVE

The objective of this report is to exhibit automatic A/C controller unit that maintain requested temperature using Arduino equipment.

- PART LIST

  - Arduino Uno R3 Device

  - USB cable

  - Jumper wires

  - Breadboard 3

  - LEDs (red, blue, yellow)

  - Adjustable Potentiometer

  - DS18B20 Temperature Sensor

  - I2C LCD Display

- BACKGROUND

3.1. Arduino Uno

- In this project, I used Arduino Uno R3.

- It has 2 power source pins 3.3v and 5v in addition to 3 grounds.

- Also, it has 6 pins for analogy signal.

- In addition, there is 14 digital pins with 5 of them are PWM pins.

3.2. Temperature Sensor

- I used DS18B20 Temperature Sensor model.

- It has 3 wires. Left connected to the ground, Right to the voltage source. The middle connected to both power source using resistor and to digital to read it input.

- It can read operating temperature range: -55ºC to +125ºC, then it converts it to digital value.

3.3. I2C LCD Display

3.5 Programing (code)

- Function

  - Serial.begin(X);    // Start serial communication with X speed

  - Serial.print();	// Print text or value to the console

  - analogRead (AX)	// Read analogy value from analogy pin X.

  - analogWrite(X, Y)// Write Y value to PWM pin X.

  - delay(X)		// Add some delay to code my X ms

  - sensors.begin(); 	// Start up the library for the sensors

- Library

  - OneWire

    - Used to connect temperature sensor.

  - DallasTemperature

    - Used to transform voltage temperature sensor value taken from the pin to Celsius value.

  - LiquidCrystal

- STEPS OF PROJECT

4.0. Orginize project component

- Installing Arduino uno device above breadboard using rubber.

- Connect red(+) sign to 5v source  in both direction, and Connect blue(-) sign to ground source in both direction.

4.1. Connection and Programing Temperature Sensor

- Code

  - Downloading OneWire and DallasTemperature libraries  from Arduino program (Sketch > Include Library > Manage Library > "Search for OneWire and DallasTemperature libraries")

  - Assigning the needed code to read the temperature from sensor

- Device

  - It has 3 wires connected as follow.

    - Left connected to the ground.

    - Right to the voltage source.

    - The middle connected to both power source using resistor and to digital pin 13.

4.2. Reading User Temperature Request by Potentiometer

- Code

  - Making A0 as analogy input from Potentiometer

  - After Reading the analog value (0-1023) do this equation to it get a range of value between 18-33

    - Requested temperature = (A0/64) + 18

- Device

  - It has 3 wires connected as follow.

    - Left connected to the ground.

    - Right to the voltage source.

    - The middle connected to Analogy pin A0.

4.3. Assigning Light to Show A/C Condition

- Code

  - Making PWM digital pins 9 and 10 as output for the blue and the red light.

  - If reading temperature are greater than the requested temperature turn on the (Cold A/C) blue light.

  - If reading temperature are lower than the requested temperature turn on the (Hot A/C) Red light.

  - Increase the (fan speed) LED light intensity comparing to the different between requested and current room temperature

- Device

  - For both LED light connect (+) long wire to the digital pins 9 and 10, and the (-) short wire to the ground using a resistor.

4.4. Configure I2C LCD Display to Show Temperature Condition

4.5. Overall Design